AA Portable Power Corp.

SPECIFICATION FOR

MODEL NO. : H00450018-US-1

1.8 WATTS CHARGER

Address：2700 Rydin Rd. Unit C Richmond CA 94804

TEL：1-800-448-4428, 510-525-4710

FAX： 510-525-4718

| DESCRIPTION |  | SPECIFICATION |  | HCT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MODEL NO. |  | H00450018-US-1 |  | DOCUMENT NO. |  |  |
| PREPARED BY | DESIGNED BY |  | CHECKED BY | APPROVED BY | ISSUED DATE | 12-02-2005 |
| HEQINGQIANG |  |  |  |  | REV. | A |

- Revision History

| Rev | Description | Date |
| --- | --- | --- |
| A0 | Original | 2005.12.2 |

1.0 INPUT	4

1.1  Voltage	4

1.2  Frequency	4

1.3  Current	  .4

1.4  Power efficiency	4

2.0 OUTPUT	4

2.1  charge curve	4

2.2  Led display	4

3.0 PROTECTION	4

3.1  Short circuit protection	4

4.0 ENVIRONMENT:	4

4.1  Ambient operation temperature	4

4.2  Ambient operation relative humidity	4

4.3  Ambient storage temperature	4

4.4  Ambient storage relative humidity	4

5.0 MTBF	5

6.0 DIMENSION.………………………………………………………………5

# 1.0 INPUT

## 1.1 Voltage

| Minimum | Normal | Maximum | Unit |
| --- | --- | --- | --- |
| 100 | 115/230 | 240 | Vrms |

## 1.2 Frequency

47Hz~63Hz

## 1.3 Current

0.4A.(MAX)

## 1.4  Power efficiency

80% (min.) at full load, 115Vac 60Hz.

# 2.0 OUTPUT

## 2.1  Charge curve

Charger current(A)

0.4A

4.5V

Voltage(V)

2.1.1  This charger can charge RCR123  Li-ion battery

2.1.2  End charge by 1/10C detection

## 2.2  Led display

## Red on charging

Green on full charge

# 3.0 PROTECTION

## 3.1 Short circuit protection

The output short to ground, it will auto-recovery without damage.

# 4.0 ENVIRONMENT:

## 4.1 Ambient operation temperature

0℃ to  +40℃

## 4.2 Ambient operation relative humidity

20%  to  85%

## 4.3 Ambient storage temperature

-40℃ to +70℃

## 4.4 Ambient storage relative humidity

10%  to  95%

# 5.0 MTBF

40,000 hours base on  bellcore  TR332 document  required  at 25℃.

# 6.0 Dimension

Outline dimension:84.0L X 46.0W X 28.5(H) mm MAX;